AROYA · CULTIVATION REPORT Vangarden — 2-Week Cultivation Report Facility 4537 · Window 2026-04-14 → 2026-04-28 · Generated 2026-04-28 Compare: 4/20 client report · 16 active flower runs · Phase-aware AROYA LED targets

| ACTIVE RUNS 16 F2–F18 · no F1 mom · no F15 | CRITICAL 7 Substrate ≥77°F · air ≥82°F · VPD ≥1.7 · EC zone ≥11 | WATCH 8 Margin warnings | IN-BAND 1 Inside phase target band |
| --- | --- | --- | --- |

Executive summary

THE HEADLINE THIS WEEK

CRITICAL FOLLOW-THROUGH Flower 6 was not resolved · the hot-room cluster expanded into early flower Flower 6 unresolved. 14 days after the 4/20 acute callout, F6 still runs 81.3°F day, VPD 1.7 day / 1.5 night, substrate avg 74.8°F max 76.6°F. Top zone EC 8.95 dS/m. With 2,064 plants it's the highest-yield-impact room in the facility — and the HVAC issue is now 14 days into a known-fault state. Hot-room cluster expanded into early flower. 4/20 cluster was F6/F10/F16/F18. Today every Week 1–2 room runs 80–83°F day with substrate 75–79°F. F8 D7 (sub max 78.5°F), F17 D5 (83.3°F night, sub max 78.8°F), F13 D12 (sub zone max 80.9°F), F16 D14, F10 D19. The most yield-sensitive rooms in the facility being run with degraded root function. Pore-EC stack widened. 4/20 flagged F12 z72540 at 10.78. Today four rooms past 10 dS/m: F2 z72469=12.05, F12 z72538=10.17, F14 z72552=10.32, F9 z72525=10.09. F8 / F17 night-temp inversion. Both running hotter at night than day. HVAC isn't holding setpoints in the dark cycle.

WHAT'S HOLDING F9 generative pivot landed · ripening rooms clean F9 generative tap landed. VWC 60.8% → 48.2%, dryback opened to 18.9% (best in facility), EC ramped 6.39 → 7.3. The 4/20 plan worked. Hot-spot z72525 still hot at 10.09 — physical inspection still warranted, but room-level steering is on. Cleanest finishes. F5 (D54 ripening, 53.9% VWC, 75.4°F day, EC 6.1), F12 (D49 ripening, 50.4% VWC, 74.0°F day), F14 (D47, 74.6°F, dryback 15.5%).

What changed vs the 2026-04-20 report

Direct comparison against the snapshot from the 4/20 client meeting (Bryan Gay · G. Morales · E. Kokkennen). Numbers are 24h facility averages then vs 14-day day-mean now — directionally correct, not apples-to-apples on window length.

| Room | Phase | Air °F (Δ vs 4/20) | VPD (Δ) | VWC % (Δ) | What moved |
| --- | --- | --- | --- | --- | --- |
| Flower 2 D55 · CRITICAL | Week 8 Generative | 74.8°F (+1.1) | 1.31 (+0.18) | 50.4% (-3.1) | Late ripening. z72469 EC climbed to 12.05 — extreme. |
| Flower 3 D28 · WATCH | Bulking/Vegetative 2 | 79.5°F (+2.6) | 1.41 (+0.07) | 48.4% (-5.4) | Hotter (+1.7°F). EC climbing on schedule. |
| Flower 4 D40 · OK | Bulking/Vegetative 2 | 77.4°F (+0.4) | 1.09 (+0.05) | 47.2% (+10.4) | More in-band; cooled slightly. EC 5.3 — on track. |
| Flower 5 D54 · WATCH | Ripening/Generative 2 | 75.4°F (-0.2) | 1.37 (-0.02) | 53.9% (+15.9) | Reset — likely new flip mid-window. VWC 38% → 53.9%. |
| Flower 6 D39 · CRITICAL | Bulking/Vegetative 2 | 81.3°F (-0.2) | 1.66 (-0.06) | 50.6% (+15.3) | NOT RESOLVED. 14-day day mean 81.3°F vs 81.5°F on 4/20. |
| Flower 7 D42 · WATCH | Week 6 Generative | 80.4°F (+5.6) | 1.53 (+0.47) | 51.0% (+1.6) | Hotter (+5.6°F day) but VPD held. Possible photoperiod shift. |
| Flower 8 D7 · CRITICAL | Week 1 Generative | 82.2°F (+9.7) | 1.39 (+0.55) | 37.7% (+35.3) | NEW FLIP 4/21. 82.2 / 82.4°F (no DIF), substrate 77–78.5°F in Week 1. |
| Flower 9 D40 · WATCH | Bulking/Vegetative 2 | 76.3°F (+1.3) | 1.19 (+0.30) | 48.2% (-12.5) | GENERATIVE TAP LANDED. VWC 60.8 → 48.2, dryback 18.9%, EC up. |
| Flower 10 D19 · CRITICAL | Week 2 Generative | 80.2°F (+0.0) | 1.52 (+0.10) | 40.8% (+12.2) | Sub max 78.1°F. VPD/VWC ratchet correctly. |
| Flower 11 D21 · WATCH | Bulking/Vegetative 2 | 79.5°F (+1.6) | 1.31 (-0.16) | 44.2% (-1.1) | Steady; D21 late veg-phase irrigation. Sub max 75.8°F at threshold. |
| Flower 12 D49 · WATCH | Ripening/Generative 2 | 74.0°F (-2.5) | 1.18 (-0.20) | 50.4% (-8.5) | Finishing ripening cleanly. EC 10.17 z72538 still in the system. |
| Flower 13 D12 · CRITICAL | Week 1 Generative | 79.2°F (+4.1) | 1.47 (+0.46) | 44.0% (-1.8) | NEW FLIP 4/16. Sub zone max 80.9°F in Week 1 — worst root-zone heat. |
| Flower 14 D47 · WATCH | Week 6 Generative | 74.6°F (-1.9) | 1.24 (-0.18) | 50.0% (+16.6) | Calm late-flower. z72552 EC 10.32 — flush-or-finish call. |
| Flower 16 D14 · CRITICAL | Week 2 Generative | 81.6°F (+1.1) | 1.41 (+0.33) | 42.3% (-0.4) | NEW FLIP 4/14. Sub max 78.2°F in Week 2. |
| Flower 17 D5 · CRITICAL | Week 1 Generative | 82.5°F (+6.7) | 1.63 (0.00) | 38.9% (+35.8) | NEW FLIP 4/23. Air 82.5 day / 83.3 night — DIF reversed. Sub max 78.8°F. |
| Flower 18 D26 · WATCH | Bulking/Vegetative 2 | 80.6°F (+0.5) | 1.36 (+0.07) | 48.2% (+10.6) | D26 still hot (sub max 77.6°F). VPD held at 1.4 — irrigation compensating. |

Hot-room cluster — F6 · F8 · F10 · F13 · F16 · F17 · F18

The pattern.  Air temp ≥80°F day combined with substrate ≥75°F (uptake degrading), with zone temps reaching 78–81°F (root-function red line). Consistent across rooms that flipped recently AND across F6 which has been hot for 30+ days — meaning the heat source is the rooms / their HVAC, not just lighting load on a young canopy.

| Room | Day | Phase | Air d/n (DIF) | Sub avg/max°F | Notes |
| --- | --- | --- | --- | --- | --- |
| Flower 17 | D5 | Week 1 Generative | 82.5 / 83.3°F (-0.8°F) | 77.6 / 78.8°F | Hottest early flower in the facility. 5 days flowering in roots routinely 80°F. Generative-phase EC ramp won't land if uptake is degraded. DAY-1 PRIORITY. |
| Flower 8 | D7 | Week 1 Generative | 82.2 / 82.4°F (-0.2°F) | 77.4 / 78.5°F | Same pattern as F17, 2 days older. Will run into stretch overshoot — early-flower 82°F day temps push internodal length without VPD ceiling that mid-flower can absorb. |
| Flower 13 | D12 | Week 1 Generative | 79.2 / 79.8°F (-0.6°F) | 75.0 / 79.0°F | Worst single substrate-zone reading in facility (z72544 = 80.9°F). Worth physical inspection — sensor placement vs heat source, dripper proximity to a duct. |
| Flower 16 | D14 | Week 2 Generative | 81.6 / 81.7°F (-0.1°F) | 76.8 / 78.2°F | Two weeks in, hot canopy with hot roots. Dryback 18.1% (good) — but steering signal applied to a degraded root system. |
| Flower 6 | D39 | Bulking/Vegetative 2 | 81.3 / 79.8°F (+1.5°F) | 74.8 / 76.6°F | UNRESOLVED from 4/20. Same room, same problem, 14 days later. Largest plant count in facility (2,064) so highest yield-impact room. |
| Flower 10 | D19 | Week 2 Generative | 80.2 / 79.2°F (+1.0°F) | 75.8 / 78.1°F | Flagged on 4/20 and still warm. Mid-bulking can absorb heat for a while but EC will compress the top of the run. |
| Flower 18 | D26 | Bulking/Vegetative 2 | 80.6 / 79.7°F (+0.9°F) | 75.9 / 77.6°F | Flagged on 4/20 and still warm. Sub max 77.6°F. VPD held at 1.4 — irrigation compensating, but root temp persists. |

Pattern conclusion

F6 plus every just-flipped room running 76–79°F substrate (one zone at 81°F) means the issue is structural. Either:

HVAC capacity is undersized for the current crop schedule (more rooms in early flower at once = more lighting heat, exceeding cooling),

Return-air paths are fighting with each other (the 4/20 dehumidification CapEx note is the pre-existing trail on this), or

Substrate-side heating — drippers near radiant heat, slabs in sun, chiller loop unbalanced — only the F13 z72544 reading would localize this.

Diagnostic step: check whether the air-temp setpoint on the BMS / Argus matches what AROYA's H421 sees. The 4/20 carry-over flagged a possible Argus-vs-AROYA mismatch in F6; if a 6°F mismatch exists in F6 it likely exists facility-wide.

TODAY'S TOPIC

Root-zone temperature is the yield ceiling

This is a different story than 4/20's "generative pivot mid-cycle." That conversation was about steering the plant. This one's about whether the plant can do anything with the steering at all.

The biology, briefly

65–75°F substrate is the root-function plateau. Nutrient uptake, root respiration, water transport — all running at design.

75–77°F — uptake of K, Mg, Ca starts degrading. The plant can still flower but salt-stack risk rises (more EC at the probe than the plant is actually drinking).

≥77°F — root function meaningfully degraded. Pythium / fungal risk rises. EC rapidly decouples from plant uptake.

≥80°F — root system in damage mode. Stunted lateral growth, bronzing, loss of internode pull-down.

Why early flower is the worst place to have this

Early flower is when the plant is supposed to receive the generative signal — fewer / larger irrigation shots, EC ramping from 3 → 6, substrate concentration intentionally rising. The plant tolerates this because root function is full-throttle. If roots are at 78°F, the EC ramp that's supposed to drive the bulking-phase reservoir instead just stacks at the probe. Six weeks later you read 8 dS/m at the sensor and say "EC is on schedule" — but the plant only got 6.

What to do this week

| OPERATIONAL MITIGATIONS · 3–7 DAYS Check Argus / BMS reading vs AROYA H421 in F6, F8, F13, F16, F17. If AROYA reads 6°F higher than the BMS, HVAC isn't being asked to cool to the right setpoint. Drop night setpoint 2°F in early-flower rooms. Even if HVAC can't hold it, you'll buy a 1°F real drop — and reverse the night-hotter inversion in F8 / F17. Push P1 earlier in early-flower rooms. A larger AM shot at lights-on cools the substrate via evaporation and resets the dryback ceiling before the room heats up. Cut feed EC by 0.3–0.5 in early flower until substrate temp comes down. You won't get the EC ramp anyway with degraded uptake; lower feed reduces the salt stack at the probe. Audit zone 72544 (F13) physically. 80.9°F is either a localized heat source or a probe placement issue. Cheapest fix in the facility. | STRUCTURAL FIXES · MULTI-WEEK HVAC capacity audit — is dehumid / cooling sized for current simultaneous-flower-room count? Cluster grows when more rooms enter flower. Return air balancing in the hot rooms. F6 has been hot 30+ days; either it always was or something shifted. Substrate-side cooling — chilled water, slab cooling, or just isolating drippers from radiant heat. Crop schedule sequencing — if HVAC is the limit, splitting flips so 6 rooms aren't simultaneously in Week 1–2. The 4/20 dehumidification CapEx conversation is the pre-existing trail. Worth pulling that quote back up against today's data. |
| --- | --- |

Room-by-room (14-day window)

Phase target legend: green inside band · amber within 15% of edge · red outside. Day/night split assumes lights-on 06:00 PDT (13:00 UTC).

| Room | Status | Phase / Day | Air d/n °F | DIF | VPD d/n | VWC% | EC | Sub avg/max °F | Dryback |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flower 2 | CRITICAL | D55 Week 8 Gen | 74.8/75.2 | -0.4 | 1.31/1.38 | 50.4 | 7.02 | 70.3/71.4 | 13.0% |
| Flower 3 | WATCH | D28 Bulking/Veg 2 | 79.5/79.4 | +0.1 | 1.41/1.42 | 48.4 | 5.15 | 73.6/75.5 | 14.3% |
| Flower 4 | OK | D40 Bulking/Veg 2 | 77.4/76.3 | +1.1 | 1.09/1.00 | 47.2 | 5.34 | 73.4/74.5 | 15.9% |
| Flower 5 | WATCH | D54 Ripening/Gen 2 | 75.4/74.1 | +1.4 | 1.37/1.11 | 53.9 | 6.11 | 70.5/71.6 | 15.9% |
| Flower 6 | CRITICAL | D39 Bulking/Veg 2 | 81.3/79.8 | +1.5 | 1.66/1.47 | 50.6 | 6.28 | 74.8/76.6 | 16.2% |
| Flower 7 | WATCH | D42 Week 6 Gen | 80.4/80.2 | +0.1 | 1.53/1.52 | 51.0 | 6.47 | 74.8/76.0 | 13.1% |
| Flower 8 | CRITICAL | D7 Week 1 Gen | 82.2/82.4 | -0.2 | 1.39/1.44 | 37.7 | 3.80 | 77.4/78.5 | 12.3% |
| Flower 9 | WATCH | D40 Bulking/Veg 2 | 76.3/76.2 | +0.1 | 1.19/1.22 | 48.2 | 7.26 | 71.8/73.7 | 18.9% |
| Flower 10 | CRITICAL | D19 Week 2 Gen | 80.2/79.2 | +1.0 | 1.52/1.11 | 40.8 | 5.67 | 75.8/78.1 | 16.7% |
| Flower 11 | WATCH | D21 Bulking/Veg 2 | 79.5/80.0 | -0.5 | 1.31/1.49 | 44.2 | 5.45 | 75.1/75.8 | 16.1% |
| Flower 12 | WATCH | D49 Ripening/Gen 2 | 74.0/75.7 | -1.6 | 1.18/1.58 | 50.4 | 6.74 | 70.6/70.9 | 17.4% |
| Flower 13 | CRITICAL | D12 Week 1 Gen | 79.2/79.8 | -0.6 | 1.47/0.95 | 44.0 | 3.56 | 75.0/79.0 | 10.4% |
| Flower 14 | WATCH | D47 Week 6 Gen | 74.6/75.0 | -0.4 | 1.24/0.92 | 50.0 | 6.13 | 70.7/71.4 | 15.5% |
| Flower 16 | CRITICAL | D14 Week 2 Gen | 81.6/81.7 | -0.1 | 1.41/1.44 | 42.3 | 4.65 | 76.8/78.2 | 18.1% |
| Flower 17 | CRITICAL | D5 Week 1 Gen | 82.5/83.3 | -0.8 | 1.63/1.65 | 38.9 | 2.85 | 77.6/78.8 | 13.4% |
| Flower 18 | WATCH | D26 Bulking/Veg 2 | 80.6/79.7 | +0.9 | 1.36/1.25 | 48.2 | 6.01 | 75.9/77.6 | 16.0% |

Per-room reads

Flower 2 [CRITICAL] D55 · Week 8 Generative · 1764 plants · flip 2026-03-04 · harvest 2026-05-03 Read. EC zone 12.1 Hot zones. Pore EC: z72469=12.05 · z72495=11.28 · z72496=9.36 dS/m Substrate temp: z72498=74.2°F · z72468=72.2°F · z72496=71.7°F

Flower 3 [WATCH] D28 · Bulking/Vegetative 2 · 1836 plants · flip 2026-03-31 · harvest 2026-05-30 Read. sub max 75.5°F Hot zones. Pore EC: z72500=7.94 · z72499=6.70 · z72501=6.62 dS/m Substrate temp: z72499=77.1°F · z72501=76.7°F · z72470=76.1°F

Flower 4 [OK] D40 · Bulking/Vegetative 2 · 1836 plants · flip 2026-03-19 · harvest 2026-05-18 Read. All key metrics inside phase band — no action needed. Hot zones. Pore EC: z72503=8.18 · z72472=7.67 · z72473=7.21 dS/m Substrate temp: z72473=75.7°F · z72503=75.7°F · z72506=75.4°F

Flower 5 [WATCH] D54 · Ripening/Generative 2 · 1744 plants · flip 2026-03-05 · harvest 2026-05-04 Read. EC zone 9.9 Hot zones. Pore EC: z72510=9.92 · z72509=8.83 · z72475=7.09 dS/m Substrate temp: z72474=72.6°F · z72510=72.5°F · z72508=71.9°F

Flower 6 [CRITICAL] D39 · Bulking/Vegetative 2 · 2064 plants · flip 2026-03-20 · harvest 2026-05-19 Read. 4/20 unresolved (chronic) · sub max 76.6°F · air 81.3°F · VPD 1.66 Hot zones. Pore EC: z72514=8.95 · z72476=8.44 · z72477=8.13 dS/m Substrate temp: z72513=78.1°F · z72476=77.8°F · z72514=77.7°F

Flower 7 [WATCH] D42 · Week 6 Generative · 1650 plants · flip 2026-03-17 · harvest 2026-05-16 Read. sub max 76.0°F · air 80.4°F · VPD 1.53 Hot zones. Pore EC: z72479=8.33 · z72515=8.20 · z72478=8.04 dS/m Substrate temp: z72516=77.1°F · z72479=76.6°F · z72478=76.4°F

Flower 8 [CRITICAL] D7 · Week 1 Generative · 1630 plants · flip 2026-04-21 · harvest 2026-06-20 Read. sub max 78.5°F · sub avg 77.4°F · air 82.2°F Hot zones. Pore EC: z72521=5.57 · z72520=4.81 · z72481=4.64 dS/m Substrate temp: z72481=79.8°F · z72521=79.6°F · z72519=79.0°F

Flower 9 [WATCH] D40 · Bulking/Vegetative 2 · 1272 plants · flip 2026-03-19 · harvest 2026-05-18 Read. EC zone 10.1 Hot zones. Pore EC: z72525=10.09 · z72526=9.55 · z72482=8.34 dS/m Substrate temp: z72526=75.3°F · z72527=74.7°F · z72525=74.3°F

Flower 10 [CRITICAL] D19 · Week 2 Generative · 1650 plants · flip 2026-04-09 · harvest 2026-06-08 Read. sub max 78.1°F · sub avg 75.8°F · air 80.2°F · VPD 1.52 Hot zones. Pore EC: z72532=8.34 · z72531=7.70 · z72529=6.81 dS/m Substrate temp: z72529=79.3°F · z72483=78.4°F · z72528=78.3°F

Flower 11 [WATCH] D21 · Bulking/Vegetative 2 · 1650 plants · flip 2026-04-07 · harvest 2026-06-06 Read. sub max 75.8°F · sub avg 75.1°F Hot zones. Pore EC: z72533=8.10 · z72484=7.69 · z72537=7.43 dS/m Substrate temp: z72534=76.4°F · z72535=76.4°F · z72484=76.3°F

Flower 12 [WATCH] D49 · Ripening/Generative 2 · 1272 plants · flip 2026-03-10 · harvest 2026-05-09 Read. EC zone 10.2 Hot zones. Pore EC: z72538=10.17 · z72540=9.39 · z72539=8.73 dS/m Substrate temp: z72539=72.2°F · z72538=72.1°F · z72541=71.7°F

Flower 13 [CRITICAL] D12 · Week 1 Generative · 1800 plants · flip 2026-04-16 · harvest 2026-06-15 Read. sub max 79.0°F Hot zones. Pore EC: z72486=7.95 · z72544=6.35 · z72545=4.46 dS/m Substrate temp: z72544=80.9°F · z72543=80.6°F · z72546=79.2°F

Flower 14 [WATCH] D47 · Week 6 Generative · 1496 plants · flip 2026-03-12 · harvest 2026-05-11 Read. EC zone 10.3 Hot zones. Pore EC: z72552=10.32 · z72551=8.99 · z72549=8.19 dS/m Substrate temp: z72548=72.9°F · z72552=72.3°F · z72487=72.3°F

Flower 16 [CRITICAL] D14 · Week 2 Generative · 1836 plants · flip 2026-04-14 · harvest 2026-06-13 Read. sub max 78.2°F · sub avg 76.8°F · air 81.6°F Hot zones. Pore EC: z72557=7.06 · z72553=6.72 · z72555=6.00 dS/m Substrate temp: z72554=79.9°F · z72488=79.6°F · z72553=78.7°F

Flower 17 [CRITICAL] D5 · Week 1 Generative · 1178 plants · flip 2026-04-23 · harvest 2026-06-22 Read. sub max 78.8°F · sub avg 77.6°F · air 82.5°F · VPD 1.63 Hot zones. Pore EC: z72648=4.06 · z72644=3.87 · z72647=3.78 dS/m Substrate temp: z72644=79.9°F · z72645=79.5°F · z72646=79.4°F

Flower 18 [WATCH] D26 · Bulking/Vegetative 2 · 1488 plants · flip 2026-04-02 · harvest 2026-06-01 Read. sub max 77.6°F · sub avg 75.9°F · air 80.6°F · EC zone 9.4 Hot zones. Pore EC: z72651=9.38 · z72650=8.68 · z72490=7.32 dS/m Substrate temp: z72650=78.6°F · z72653=78.4°F · z72651=78.3°F

Carry-overs from prior sessions

| Argus / BMS vs AROYA temperature mismatch | RECONFIRMED | F6 has not cooled in 14 days, and every newly-flipped room runs 80–82°F. AROYA and the BMS likely reading different setpoints. Highest-leverage diagnostic action this week. |
| --- | --- | --- |
| Dehumidification CapEx | STILL APPLICABLE | Hot-room cluster expanding as more rooms enter early flower simultaneously is exactly what a HVAC capacity bottleneck looks like. |
| Strawberry Glue / McGilligoo foxtailing | UNVERIFIED | No cultivar→room mapping in this pull. Worth asking which of F11 (D21) and F14 (D47, EC z72552=10.32) are running those genetics. F2 D55 z72469=12.05 is also a foxtailing-prone phase. |
| 1-gal pot dryback behavior | PARTIAL READ | F8 D7 dryback 12.3%, F13 D12 dryback 10.4%, F17 D5 dryback 13.4% — early-flower drybacks shallow but these are also new flips. More diagnostic at D14+. F9's 18.9% dryback shows what a healthy bulking dryback looks like. |
| F9 generative pivot (4/20 recommendation) | LANDED | VWC 60.8 → 48.2, dryback opened to 18.9% (best in facility), EC ramped 6.39 → 7.3. The plan worked. Hot-spot z72525 still hot at 10.09 — physical inspection still warranted. |
| F12 ripening EC drift | CONTINUING | z72540 was 10.78; today z72538 = 10.17, z72540 = 9.39. Ripening progressed without explicit flush. Harvest 5/9 at D49 — drift will end on its own, but watch for tip burn / early senescence in high-EC zones. |
| Flower 6 escalation (4/20 acute) | UNRESOLVED | See hot-room cluster section. |
| 1.5-gal trial / plastic pots / reduced plant count / two drippers / plenum fans / 2°F night drop / 20–25% dryback on Strawberry Glue | STATUS UNKNOWN | Carrying these forward; please correct anything resolved or superseded. |

Methods & caveats

| DATA SOURCES AROYA MCP — list_rooms, spa_list_harvest_groups, get_facility_overview, get_room_history (14-day per-zone hourly bins, 16 flower rooms × ~41 series per room). 14 days of 1h-binned mean/min/max for soil_moist, soil_temp, pore_ec, air_temp, vpd, rel_hum, co2, abs_hum. Comparison baseline: 2026-04-20 client report (24h facility-overview averages from that date). Photoperiod: lights-on 06:00 PDT (13:00 UTC) → off 18:00 PDT. Day/night UTC hours documented; transition hours excluded. | NOT IN SCOPE Cultivar → room mapping. spa_list_harvest_groups returned cultivar IDs but not names. Strawberry Glue / McGilligoo follow-through gated on this. Per-device battery / RSSI / gap statistics — prior 4/20 device fleet inventory (623 devices) still applies barring fleet changes. Yield context for finishing rooms (F2 / F5 / F12). Dry weights vs targets not in this scope. Drip / drain EC differential (dfi / dfi_drain / pore_ec_drip / pore_ec_drain) — diagnostic for whether EC stack is feed-side or substrate-side. Not pulled this session. |
| --- | --- |

Phase targets used

AROYA crop-steering target table (LED). Early flower (Wk1–3): VPD 0.7–1.0 day, dryback 25–40%, EC 3–8 ramping. Mid bulking (Wk4–5): VPD 1.0–1.2, dryback 15–30%, EC 6–8. Late flower (Wk6–7): VPD 0.9–1.3, dryback 20–35%, EC 6–7. Ripening (Wk8–9): VPD 1.0–1.3, dryback 30–45%, EC 7–9. Substrate temp 65–75°F all phases (75°F threshold, 77°F red line).

Generated 2026-04-28  ·  Vangarden facility 4537  ·  14-day window  ·  16 active flower runs  ·  phase-aware AROYA LED targets